ZBAITU_ENGRAVER_O说明书

一、概述

1. 框架说明

升降台

激光模组

SD卡插槽

USB接口

电源接口

Y轴电机

X轴电机

Y轴电机

- 电源接口:连接电源，给机器供电

- USB接口: 连接电脑传输数据，通过电脑操控机器

- SD卡插槽:储存GCODE代码，完成脱机雕刻

- X、Y轴电机: 控制激光精准的移动，完成雕刻

- 升降台:调节激光模组与雕刻物体间的距离，以达到最佳焦距

- 激光模组:发出激光进行雕刻或者切割

2.配件清单

| 配件 | 描述 |
| --- | --- |
|  | USB 数据线：若要通过电脑连接机器，请将USB数据线的⼀端连接到机器的 USB连接⼝，另⼀段连接到电脑的USB接⼝上。 |
|  | 12V-6A电源适配器：若要使⽤机器，请将适配器连接到机器电源接⼝，将插 头连接到适配器，并将适配器插⼊交流电源插座。 |
|  | 防护眼镜：雕刻机⼯作时请勿直视激光，若要观查雕刻情况与过程，请佩戴防 护眼镜。 |

二、组装机器

1.配件

| 配件图 | 名称 | 规格 | 数量 |
| --- | --- | --- | --- |
|  | L型角槽 |  | 4 |
|  | 手拧螺母 | M5 | 8 |
|  | 螺丝 | M5*10 | 9 |
|  | 角槽螺母 | M5 | 1 |
|  | 螺丝 | M3*8 | 2 |
|  | 不锈钢垫片 | M5*14*1 | 4 |

2.组装步骤

- 组装Y轴滑块

- 柱状整体框架

- 安装支撑脚

- 穿入同步带

- 固定同步带（注意皮带松紧度）

- 将YR轴线材卡入型材槽

- 安装激光线及X轴

- 安装及固定X,Y轴线材

- 安装激光模组，并连接激光模组到主板

11.固定X轴螺丝

2. 连接软件

连接方式1（有线）：

- 接通机器电源

- 将我们提供的数据线一端连接机器USB连接口，一端连接电脑USB口

- 打开电脑操作软件，点击连接按钮，即可连接

连接方式2（蓝牙）：

- 接通机器电源

- 打开手机或电脑的WIFI连接列表

- 选择网络"GRBL_ESP"，密码:123456789.（PC 请断开有线网络），等待网络连接

- 然后打开浏览器在地址栏输入 控制器的 IP 地址（示例中为“192.168.0.1”）打开 Web 控制界面

- 按下图设置参数

  - 设置广播模式位-Bluetooth

- 设置完成后，重启

- 打开电脑，查找蓝牙btgrblesp，并连接。

- 打开操作软件，选择对应串口点击连接按钮，即可连接

连接方式3（WIFI_AP模式）：

- 接通机器电源

- 打开手机或电脑的WIFI连接列表

- 选择网络"GRBL_ESP"，密码:123456789.（PC 请断开有线网络），等待网络连接

- 然后打开浏览器在地址栏输入 控制器的 IP 地址（示例中为“192.168.0.1”）打开 Web 控制界面

链接方式4（WIFI_STA模式）：

- 接通机器电源

- 打开手机或电脑的WIFI连接列表

- 选择网络"GRBL_ESP"，密码:123456789.（PC 请断开有线网络），等待网络连接

- 然后打开浏览器在地址栏输入 控制器的 IP 地址（示例中为“192.168.0.1”）打开 Web 控制界面

- 按下图设置参数

  - 设置WiFi名称

  - 设置WiFi密码

  - 设置控制板在路由同段内地址

  - 设置Telnet参数

- 设置完成后，重启，然后打开浏览器在地址栏输入 控制器的 IP 地址（示例中为“192.168.31.127”）打开 Web 控制界面

三、注意事项

1. 安全事项

- 禁止直接照射人体或动物的眼睛，否则会对眼睛造成伤害！

- 禁止照射人体、动物、衣物、贵重物品等，否则会造成伤害或损坏！

- 禁止给儿童或没有操作能力的人使用！

- 禁止在有易燃易爆物品或可燃气体的场所使用，以免造成火灾或爆炸！

- 机器雕刻时禁止直视激光焦点，如需观察，请佩戴防护眼镜！

- 禁止长时间注视激光焦点，否则将会影响您的视力！

- 不使用机器时请把掉电源。

2.  雕刻须知

- 在开始雕刻前注意焦距是否准确！

  - 30W、40W-PRO可调焦模组，镜头与雕刻物距离为20-40mm

  - 40W-PRO定焦20mm镜头与雕刻物距离为20mm，此模组雕刻佳

  - 40W-PRO定焦40mm镜头与雕刻物距离为40mm，此模组切割佳

  - 80W定焦45mm镜头与雕刻物距离为45mm，此模组切割最佳

- 理论上机器可长时间工作，但为了更好的保护激光头延长其使用寿命，建议工作2小时休息2分钟

- 如有任何疑问可联系客服（联系方式），也可通过网页"联系我们"发送邮件联系我们。

四、维护保养

- 请勿在潮湿/高温的场合使用机器。

- 请勿用潮湿的手触摸操作机器。

- 机器的镜头请定时用棉棒蘸取酒精擦拭，有利于保护镜头。

五、联系我们

- 企业名称：ZBAITU

- 地址:深圳市龙华区民治街道工业西路勃兴工业区C栋501

- Email：zbaitu@zbaitu-tech.com

附录

| TSLD40W_XX_PRO |  |  |  |
| --- | --- | --- | --- |
| Material | Marking | Craving | Cutting |
| Plywood 5mm | √ | √ | √ |
| Plywood 8mm | √ | √ | √ |
| Plywood 18mm | √ | √ | × |
| MDF board 5mm | √ | √ | √ |
| MDF board 8mm | √ | √ | √ |
| MDF board 18mm | √ | √ | × |
| Balsa 8mm | √ | √ | √ |
| Hard wood 18mm | √ | √ | × |
| Acrylic 8mm(Dark Color) | √ | √ | √ |
| Brushed Acrylic(Transparent) | √ | × | × |
| Corrugated cardboard < 8mm | √ | √ | √ |
| Kraft Paper 0.5mm | √ | √ | √ |
| Paperboard 2mm | √ | √ | √ |
| Weeding invitation paper 0.5mm | √ | √ | √ |
| Colored A4 Paper(Not White) | √ | √ | √ |
| A4 Paper(White) | × | × | √ |
| Spray art stencils from mylar sheets(<1mm) | × | × | √ |
| Heat transfer vinyl | × | × | √ |
| bamboo | √ | √ | √ |
| Stamp rubber | √ | √ | √ |
| Non-woven | √ | × | √ |
| Cotton | √ | × | √ |
| Linen | √ | × | √ |
| Velvet | √ | × | √ |
| Canvas | √ | × | √ |
| Fabric polyester | √ | × | √ |
| Fabric synthetics | √ | × | √ |
| Leather < 2mm | √ | √ | √ |
| Leather < 4mm | √ | √ | √ |
| Plastic | √ | √ | √ |
| PPR | √ | √ | × |
| ABS Sheets 2mm(Dark Color) | √ | √ | √ |
| PVC Sheets 2mm(Dark Color) | √ | √ | √ |
| Brushed PVC Transparent board | √ | × | × |
| Black foamex(EPE) | √ | √ | √ |
| KT Board | × | × | √ |
| EVA foam | √ | √ | √ |
| Bakelite | √ | √ | × |
| Brushed Glass | √ | × | × |
| Cobblestone | √ | × | × |
| Brushed Ceramic | √ | × | × |
| Printed Metal | √ | × | × |
| PCB borad | √ | × | × |
| Anodized Aluminium | √ | × | × |
| Brushed Stainless Steel | √ | × | × |
| Mirror Stainless Steel | √ | × | × |
| Gold | × | × | × |

| TSLD80W_FF45_PRO |  |  |  |
| --- | --- | --- | --- |
| Material | Marking | Craving | Cutting |
| Plywood 5mm | √ | √ | √ |
| Plywood 8mm | √ | √ | √ |
| Plywood 18mm | √ | √ | √ |
| MDF board 5mm | √ | √ | √ |
| MDF board 8mm | √ | √ | √ |
| MDF board 18mm | √ | √ | √ |
| Balsa 8mm | √ | √ | √ |
| Hard wood 18mm | √ | √ | √ |
| Acrylic 8mm(Dark Color) | √ | √ | √ |
| Brushed Acrylic(Transparent) | √ | × | × |
| Corrugated cardboard < 8mm | √ | √ | √ |
| Kraft Paper 0.5mm | √ | √ | √ |
| Paperboard 2mm | √ | √ | √ |
| Weeding invitation paper 0.5mm | √ | √ | √ |
| Colored A4 Paper(Not White) | √ | √ | √ |
| A4 Paper(White) | × | × | √ |
| Spray art stencils from mylar sheets(<1mm) | × | × | √ |
| Heat transfer vinyl | × | × | √ |
| bamboo | √ | √ | √ |
| Stamp rubber | √ | √ | √ |
| Non-woven | √ | × | √ |
| Cotton | √ | × | √ |
| Linen | √ | × | √ |
| Velvet | √ | × | √ |
| Canvas | √ | × | √ |
| Fabric polyester | √ | × | √ |
| Fabric synthetics | √ | × | √ |
| Leather < 2mm | √ | √ | √ |
| Leather < 4mm | √ | √ | √ |
| Plastic | √ | √ | √ |
| PPR | √ | √ | × |
| ABS Sheets 2mm(Dark Color) | √ | √ | √ |
| PVC Sheets 2mm(Dark Color) | √ | √ | √ |
| Brushed PVC Transparent board | √ | × | × |
| Black foamex(EPE) | √ | √ | √ |
| KT Board | × | × | √ |
| EVA foam | √ | √ | √ |
| Bakelite | √ | √ | × |
| Brushed Glass | √ | × | × |
| Cobblestone | √ | × | × |
| Brushed Ceramic | √ | × | × |
| Printed Metal | √ | × | × |
| PCB borad | √ | × | × |
| Anodized Aluminium | √ | × | × |
| Brushed Stainless Steel | √ | × | × |
| Mirror Stainless Steel | √ | × | × |
| Gold | × | × | × |

| Material | Cutting Speed |  |
| --- | --- | --- |
|  | TSLD40W_XX_PRO | TSLD80W_FF45_PRO |
| Plywood 2mm | 160mm/min，S1000,1Pass | 480mm/min，S1000,1Pass |
| Plywood 3mm | 120mm/min，S1000,1Pass | 480mm/min，S1000,1Pass |
| Plywood 4mm | 100mm/min，S1000,1Pass | 480mm/min，S1000,1Pass |
| Plywood 5mm | 120mm/min，S1000,2Pass | 480mm/min，S1000,1Pass |
| Plywood 8mm | 120mm/min，S1000,3Pass | 480mm/min，S1000,1Pass |
| Plywood 18mm | 120mm/min，S1000,8Pass | 480mm/min，S1000,3Pass |
| Bitch ply 3mm | 240mm/min，S1000,1Pass | 480mm/min，S1000,1Pass |
| MDF board 3mm | 120mm/min，S1000,2Pass | 480mm/min，S1000,1Pass |
| MDF board 5mm | 120mm/min，S1000,4Pass | 120mm/min，S1000,2Pass |
| MDF board 8mm | 120mm/min，S1000,5Pass | 120mm/min，S1000,3Pass |
| Black acrylic plate 3mm | 120mm/min，S1000,2Pass | 120mm/min，S1000,1Pass |
| Black acrylic plate 3mm | 120mm/min，S1000,4Pass | 120mm/min，S1000,2Pass |

| *无漆面玻璃、陶瓷 | 需要借助激光打印纸辅助雕刻 |  |
| --- | --- | --- |